Circuit drawing of Stepper Motor controller

Pattern drawing of Stepper Motor controller
(Wiring side)

Pink line shows the external parts.

Source code file of Stepper Motor controller

| 001 002 003 004 005 006 007 008 009 010 011 012 013 014 015 016 017 018 019 020 021 022 023 024 025 026 027 028 029 030 031 032 033 034 035 036 037 038 039 040 041 042 043 044 045 046 047 048 049 050 051 052 053 054 055 056 057 058 059 060 061 062 063 064 065 066 067 068 069 070 071 072 073 074 075 076 077 078 079 080 081 082 083 084 085 086 087 088 089 090 091 092 093 094 095 096 097 098 099 100 101 102 103 104 105 106 107 108 109 110 111 112 113 114 115 116 117 118 119 120 121 122 123 124 125 126 127 128 129 130 131 132 133 134 135 136 137 138 139 140 141 142 143 144 145 146 147 148 149 150 151 152 153 154 155 | ;******************************************************** ; ; Stepper Motor controller ; ; Author : Seiichi Inoue ;******************************************************** list p=pic16f84a include p16f84a.inc __config _hs_osc & _wdt_off & _pwrte_on & _cp_off ;**************** Label Definition ******************** cblock h'0c' mode ;Operation mode ;0=stop 1=right 2=left count1 ;Wait counter count2 ;Wait counter(for 1msec) endc rb0 equ 0 ;RB0 of PORTB rb1 equ 1 ;RB1 of PORTB rb2 equ 2 ;RB2 of PORTB rb5 equ 5 ;RB5 of PORTB rb7 equ 7 ;RB7 of PORTB ;**************** Program Start *********************** org 0 ;Reset Vector goto init org 4 ;Interrupt Vector clrf intcon ;Clear Interruption reg ;**************** Initial Process ********************* init bsf status,rp0 ;Change to Bank1 clrf trisa ;Set PORTA all OUT movlw b'00100111' ;RB0,1,2.5=IN RB7=OUT movwf trisb ;Set PORTB movlw b'10000000' ;RBPU=1 Pull up not use movwf option_reg ;Set OPTION_REG bcf status,rp0 ;Change to Bank0 clrf mode ;Set mode = stop clrf count1 ;Clear counter clrf count2 ;Clear counter movlw b'00000101' ;Set PORTA initial value movwf porta ;Write PORTA bsf portb,rb7 ;Set RB7 = 1 btfsc portb,rb5 ;RB5 = 0 ? goto $-1 ;No. Wait start ;************* Check switch condition ***************** btfsc portb,rb1 ;RB1(stop key) = ON ? goto check1 ;No. Next clrf mode ;Yes. Set stop mode goto drive ;No. Jump to motor drive check1 btfsc portb,rb2 ;RB2(right key) = ON ? goto check2 ;No. Next movlw d'1' ;Yes. Set right mode movwf mode ;Save mode goto drive ;No. Jump to motor drive check2 btfsc portb,rb0 ;RB0(left key) = ON ? goto drive ;No. Jump to motor drive movlw d'2' ;Yes. Set left mode movwf mode ;Save mode ;******************** Motor drive ********************* drive movf mode,w ;Read mode bz start ;mode = stop bsf portb,rb7 ;Set RB7 = 1 btfsc portb,rb5 ;RB5 = 0 ? goto $-1 ;No. Wait movlw d'5' ;Set loop count(5msec) movwf count1 ;Save loop count loop call timer ;Wait 1msec decfsz count1,f ;count - 1 = 0 ? goto loop ;No. Continue bcf portb,rb7 ;Set RB7 = 0 btfss portb,rb5 ;RB5 = 1 ? goto $-1 ;No. Wait movf porta,w ;Read PORTA sublw b'000000101' ;Check motor position bnz drive2 ;Unmatch movf mode,w ;Read mode sublw d'1' ;Right ? bz drive1 ;Yes. Right movlw b'00001001' ;No. Set Left data goto drive_end ;Jump to PORTA write drive1 movlw b'00000110' ;Set Right data goto drive_end ;Jump to PORTA write ;------- drive2 movf porta,w ;Read PORTA sublw b'000000110' ;Check motor position bnz drive4 ;Unmatch movf mode,w ;Read mode sublw d'1' ;Right ? bz drive3 ;Yes. Right movlw b'00000101' ;No. Set Left data goto drive_end ;Jump to PORTA write drive3 movlw b'00001010' ;Set Right data goto drive_end ;Jump to PORTA write ;------- drive4 movf porta,w ;Read PORTA sublw b'000001010' ;Check motor position bnz drive6 ;Unmatch movf mode,w ;Read mode sublw d'1' ;Right ? bz drive5 ;Yes. Right movlw b'00000110' ;No. Set Left data goto drive_end ;Jump to PORTA write drive5 movlw b'00001001' ;Set Right data goto drive_end ;Jump to PORTA write ;------- drive6 movf porta,w ;Read PORTA sublw b'000001001' ;Check motor position bnz drive8 ;Unmatch movf mode,w ;Read mode sublw d'1' ;Right ? bz drive7 ;Yes. Right movlw b'00001010' ;No. Set Left data goto drive_end ;Jump to PORTA write drive7 movlw b'00000101' ;Set Right data goto drive_end ;Jump to PORTA write ;------- drive8 movlw b'00000101' ;Compulsion setting drive_end movwf porta ;Write PORTA goto start ;Jump to start ;************* 1msec Timer Subroutine ***************** timer movlw d'200' ;Set loop count movwf count2 ;Save loop count tmlp nop ;Time adjust nop ;Time adjust decfsz count2,f ;count - 1 = 0 ? goto tmlp ;No. Continue return ;Yes. Count end ;******************************************************** ; END of Stepper Motor controller ;******************************************************** end |
| --- | --- |