List of all engines :

| Engine code |  | AEH | AGN | AGU |
| --- | --- | --- | --- | --- |
| Engines |  | Petrol engine | Petrol engine | Petrol engine |
| Manufactured |  | From 10.97 | From 10.97 | From 10.97 |
| Exhaust emissions fulfil |  | EU2 standard | D3 standard | D3 standard |
| Exhaust emissions warning lamp |  | no | no | no |
| No. of cylinders / valves per cylinder |  | 4 / 2 | 4 / 5 | 4 / 5 |
| Capacity |  | 1.6 | 1.8 | 1.8 |
| Output | kW at rpm | 74/5600 | 92/5900 | 110/5700 |
| Torque | Nm at rpm | 145/3800 | 170/3500 | 210/3750 to 4600 |
| Bore | Ø mm | 81.0 | 81.0 | 81.0 |
| Stroke | mm | 77.4 | 86.4 | 86.4 |
| Compression ratio |  | 10.3 | 10.3 | 9.5 |
| Injection/ignition |  | Simos 2.2 | Motronic M 3.8.5 | Motronic M 3.8.5 |
| RON | min. | 95 Super unleaded | 95 Super unleaded | 95 Super unleaded |
| Self-diagnosis |  | yes | yes | yes |
| Catalytic converter |  | yes | yes | yes |
| Lambda regulation |  | yes | yes | yes |
| Exhaust gas recirculation |  | no | no | no |
| Turbocharged |  | no | no | yes |

| Engine code |  | AGZ | AHW | AKL |
| --- | --- | --- | --- | --- |
| Engines |  | Petrol engine | Petrol engine | Petrol engine |
| Manufactured |  | From 10.97 | From 10.97 | From 10.97 |
| Exhaust emissions fulfil |  | D3 standard | EU 2 | D3 standard |
| Exhaust emissions warning lamp |  | no | no | no |
| No. of cylinders / valves per cylinder |  | 5 / 2 | 4 / 4 | 4 / 2 |
| Capacity |  | 2.3 | 1.4 | 1.6 |
| Output | kW at rpm | 110/6000 | 55/5000 | 74/5600 |
| Torque | Nm at rpm | 205/3200 | 131/3200 | 145/3800 |
| Bore | Ø mm | 81.0 | 76.5 | 81.0 |
| Stroke | mm | 90.2 | 75.6 | 77.4 |
| Compression ratio |  | 10.0 | 10.5 | 10.3 |
| Injection/ignition |  | Motronic M 3.8.3 | Magneti Marelli 4 AV | Simos 2.2 |
| RON | min. | 95 Super unleaded | 95 Super unleaded | 95 Super unleaded |
| Self-diagnosis |  | yes | yes | yes |
| Catalytic converter |  | yes | yes | yes |
| Lambda regulation |  | yes | yes | yes |
| Exhaust gas recirculation |  | no | yes | no |
| Turbocharged |  | no | no | no |

| Engine code |  | AKQ | APE | APF |
| --- | --- | --- | --- | --- |
| Engines |  | Petrol engine | Petrol engine | Petrol engine |
| Manufactured |  | From 10.97 | From 05.99 | From 05.99 |
| Exhaust emissions fulfil |  | D3 standard | D4 standard | D4 standard |
| Exhaust emissions warning lamp |  | no | no | no |
| No. of cylinders / valves per cylinder |  | 4 / 4 | 4 / 4 | 4 / 2 |
| Capacity |  | 1.4 | 1.4 | 1.6 |
| Output | kW at rpm | 55/5000 | 55/5000 | 74/5600 |
| Torque | Nm at rpm | 131/3200 | 126/3800 | 145/3800 |
| Bore | Ø mm | 76.5 | 76.5 | 81.0 |
| Stroke | mm | 75.6 | 75.6 | 77.4 |
| Compression ratio |  | 10.5 | 10.5 | 10.3 |
| Injection/ignition |  | Magneti Marelli 4 AV | Motronic ME 7.5.10 | Simos 3.3 |
| RON | min. | 95 Super unleaded | 95 Super unleaded | 95 Super unleaded |
| Self-diagnosis |  | yes | yes | yes |
| Catalytic converter |  | yes | yes | yes |
| Lambda regulation |  | yes | yes | yes |
| Exhaust gas recirculation |  | yes | yes | yes |
| Turbocharged |  | no | no | no |

| Engine code |  | APK | AQA | AQN |
| --- | --- | --- | --- | --- |
| Engines |  | Petrol engine | Petrol engine | Petrol engine |
| Manufactured |  | From 11.98 | From 09.98 | From 10.00 |
| Exhaust emissions fulfil |  | EU2 | D3 standard | EU 4 |
| Exhaust emissions warning lamp |  | no | no | yes |
| No. of cylinders / valves per cylinder |  | 4 / 2 | 4 / 5 | 5 / 4 |
| Capacity |  | 2.0 | 1.8 | 2.3 |
| Output | kW at rpm | 85/5200 | 110/5700 | 125/6200 |
| Torque | Nm at rpm | 170/2400 | 210/1750 ... 4600 | 220/3300 |
| Bore | Ø mm | 82.5 | 81.0 | 81.0 |
| Stroke | mm | 92.8 | 86.4 | 90.3 |
| Compression ratio |  | 10.5 | 9.5 | 10.5 |
| Injection/ignition |  | Motronic M 5.9.2 | Motronic ME 7.5 | Motronic ME 7.1 |
| RON | min. | 95 Super unleaded | 95 Super unleaded | 98 Super unleaded |
| Self-diagnosis |  | yes | yes | yes |
| Catalytic converter |  | yes | yes | yes |
| Lambda regulation |  | yes | yes | yes |
| Exhaust gas recirculation |  | no | no | no |
| Turbocharged |  | no | yes | no |

| Engine code |  | AQP | AQY | ARZ |
| --- | --- | --- | --- | --- |
| Engines |  | Petrol engine | Petrol engine | Petrol engine |
| Manufactured |  | From 09.99 | from 10.98 | From 09.99 |
| Exhaust emissions fulfil |  | D3 standard | D4 standard | D4 standard |
| Exhaust emissions warning lamp |  | no | no | no |
| No. of cylinders / valves per cylinder |  | 6 / 2 | 4 / 2 | 4 / 5 |
| Capacity |  | 2.8 | 2.0 | 1.8 |
| Output | kW at rpm | 150/6200 | 85/5200 | 110/5700 |
| Torque | Nm at rpm | 270/3200 | 170/240 | 210/3750 ... 4600 |
| Bore | Ø mm | 81.0 | 82.5 | 81.0 |
| Stroke | mm | 90.3 | 92.8 | 86.4 |
| Compression ratio |  | 10.5 | 10.5 | 9.5 |
| Injection/ignition |  | Motronic ME 7.1 | Motronic M 5.9.2 | Motronic ME 7.5 |
| RON | min. | 98 Super unleaded | 95 unleaded | 95 unleaded |
| Self-diagnosis |  | yes | yes | yes |
| Catalytic converter |  | yes | yes | yes |
| Lambda regulation |  | yes | yes | yes |
| Exhaust gas recirculation |  | no | no | no |
| Turbocharged |  | no | no | yes |

| Engine code |  | ATN | AUE | AUM |
| --- | --- | --- | --- | --- |
| Engines |  | Petrol engine | Petrol engine | Petrol engine |
| Manufactured |  | from 02.00 | from 02.00 | from 10.00 |
| Exhaust emissions fulfil |  | D 4 standard | EU 4 standard | EU 4 standard |
| Exhaust emissions warning lamp |  | no | yes | yes |
| No. of cylinders / valves per cylinder |  | 4 / 4 | 6 / 4 | 4 / 5 |
| Capacity |  | 1.6 | 2.8 | 1.8 |
| Output | kW at rpm | 77/5700 | 150/6200 | 110/5700 |
| Torque | Nm at rpm | 148/4500 | 270/3200 | 210/1750 ... 4600 |
| Bore | Ø mm | 76.5 | 81.0 | 81.0 |
| Stroke | mm | 86.9 | 90.3 | 86.4 |
| Compression ratio |  | 11.5 | 10.5 | 9.5 |
| Injection/ignition |  | Magneti Marelli 4LV | Motronic Me 7.1 | Motronic ME 7.5 |
| RON | min. | 98 unleaded | 98 unleaded | 95 unleaded |
| Self-diagnosis |  | yes | yes | yes |
| Catalytic converter |  | yes | yes | yes |
| Lambda regulation |  | yes | yes | yes |
| Exhaust gas recirculation |  | yes | no | no |
| Turbocharged |  | no | no | yes |

| Engine code |  | AUQ | AUS | AVU | AWP |
| --- | --- | --- | --- | --- | --- |
| Engines |  | Petrol engine | Petrol engine | Petrol engine | Petrol engine |
| Manufactured |  | from 06.01 | from 05.00 | from 07.00 | from 07.01 |
| Exhaust emissions fulfil |  | EU 4 | EU 4 | EU 4 | LEV 1 |
| Exhaust emissions lamp |  | yes | yes | yes | yes |
| No. of cylinders / valves per cylinder |  | 4 / 5 | 4 / 4 | 4 / 2 | 4 / 5 |
| Capacity |  | 1.8 | 1.6 | 1.6 | 1.8 |
| Output | kW at rpm | 132/5500 | 77/5700 | 75/3800 | 132/5500 |
| Torque | Nm at rpm | 235/1950 ... 5000 | 148/4500 | 148/3800 | 235/1950 ... 5000 |
| Bore | Ø mm | 81.0 | 76.5 | 81.0 | 81.0 |
| Stroke | mm | 86.4 | 86.9 | 77.4 | 86.4 |
| Compression ratio |  | 9.5 | 11.5 | 10.2 | 9.5 |
| Injection/ignition |  | Motronic ME 7.5 | Magneti Marelli 4 LV | Simos 3.3 | Motronic ME 7.5 |
| RON | min. | 95 unleaded | 98 unleaded | 95 unleaded | 95 unleaded |
| Self-diagnosis |  | yes | yes | yes | yes |
| Catalytic converter |  | yes | yes | yes | yes |
| Lambda regulation |  | yes | yes | yes | yes |
| Exhaust gas recirculation |  | no | yes | yes | no |
| Turbocharged |  | yes | no | no | yes |

| Engine code |  | AXP | AZD | AZH |
| --- | --- | --- | --- | --- |
| Engines |  | Petrol engine | Petrol engine | Petrol engine |
| Manufactured |  | from 05.00 | from 09.00 | from 09.00 |
| Exhaust emissions fulfil |  | EU 4 | EU 4 | EU 4 |
| Exhaust emissions lamp |  | yes | yes | yes |
| No. of cylinders / valves per cylinder |  | 4 / 4 | 4 / 4 | 4 / 2 |
| Capacity |  | 1.4 | 1.6 | 2.0 |
| Output | kW at rpm | 55/5000 | 77/5700 | 85/5400 |
| Torque | Nm at rpm | 128/3300 | 148/4500 | 170/2400 |
| Bore | Ø mm | 76.5 | 76.5 | 82.5 |
| Stroke | mm | 75.6 | 86.9 | 92.8 |
| Compression ratio |  | 10.5 | 11.5 | 10.5 |
| Injection/ignition |  | Motronic ME 7.5 | Magneti Marelli 4 MV | Motronic ME 7.5 |
| RON | min. | 95 unleaded | 98 unleaded | 95 unleaded |
| Self-diagnosis |  | yes | yes | yes |
| Catalytic converter |  | yes | yes | yes |
| Lambda regulation |  | yes | yes | yes |
| Exhaust gas recirculation |  | yes | yes | no |
| Turbocharged |  | no | no | no |

| Engine code |  | AZJ | BAD | BCA |
| --- | --- | --- | --- | --- |
| Engines |  | Petrol engine | Petrol engine | Petrol engine |
| Manufactured |  | from 05.01 | from 10.01 | from 01.02 |
| Exhaust emissions fulfil |  | EU 4 | EU 4 | EU 4 |
| Exhaust emissions lamp |  | yes | yes | yes |
| No. of cylinders / valves per cylinder |  | 4 / 2 | 4 / 4 | 4 / 4 |
| Capacity |  | 2.0 | 1.6 | 1.4 |
| Output | kW at rpm | 85/5400 | 81/5800 | 55/5000 |
| Torque | Nm at rpm | 172/3200 | 155/4400 | 126/3300 |
| Bore | Ø mm | 82.5 | 76.5 | 76.5 |
| Stroke | mm | 92.8 | 86.9 | 75.6 |
| Compression ratio |  | 10.5 | 11.5 | 10.5 |
| Injection/ignition |  | Motronic ME 7.5 | Motronic MED 7 | Motronic ME 7.5 |
| RON | min. | 95 unleaded | 98 unleaded | 95 unleaded |
| Self-diagnosis |  | yes | yes | yes |
| Catalytic converter |  | yes | yes | yes |
| Lambda regulation |  | yes | yes | yes |
| Exhaust gas recirculation |  | no | yes | yes |
| Turbocharged |  | no | no | no |

| Engine code |  | BCB | BDE | BEH |
| --- | --- | --- | --- | --- |
| Engines |  | Petrol engine | Petrol engine | Petrol engine |
| Manufactured |  | from 11.01 | from 05.01 | from 09.01 |
| Exhaust emissions fulfil |  | EU 4 | EU 4 | EU 4 |
| Exhaust emissions lamp |  | yes | yes | yes |
| No. of cylinders / valves per cylinder |  | 4 / 4 | 6 / 4 | 4 / 2 |
| Capacity |  | 1.6 | 2.8 | 2.0 |
| Output | kW at rpm | 77/5700 | 150/6200 | 85/5400 |
| Torque | Nm at rpm | 148/4500 | 270/3200 | 172/3200 |
| Bore | Ø mm | 76.5 | 81.0 | 82.5 |
| Stroke | mm | 86.9 | 90.3 | 92.8 |
| Compression ratio |  | 11.5 | 10.5 | 10.5 |
| Injection/ignition |  | Magneti Marelli 4 MV | Motronic ME 7.1.1 | Motronic ME 7.1.1 |
| RON | min. | 98 unleaded | 98 unleaded | 98 unleaded |
| Self-diagnosis |  | yes | yes | yes |
| Catalytic converter |  | yes | yes | yes |
| Lambda regulation |  | yes | yes | yes |
| Exhaust gas recirculation |  | yes | no | yes |
| Turbocharged |  | no | no | no |

| Engine code |  | BFH | BFQ | BJS | BML |
| --- | --- | --- | --- | --- | --- |
| Engines |  | Petrol engine | Petrol engine | Petrol engine | Petrol engine |
| Manufactured |  | from 06.02 | from 05.01 | from 05.02 | from 10.03 |
| Exhaust emissions fulfil |  | EU 4 | EU 4 | LEV 1 | EU 2 |
| Exhaust emissions lamp |  | yes | yes | yes | yes |
| No. of cylinders / valves per cylinder |  | 6 / 4 | 4 / 2 | 6 / 4 | 6 / 4 |
| Capacity |  | 3.2 | 1.6 | 3.2 | 3.2 |
| Output | kW at rpm | 177/6200 | 75/5600 | 177/6250 | 177/6250 |
| Torque | Nm at rpm | 320/3200 | 148/3800 | 320/2800-3200 | 320/2800-3200 |
| Bore | Ø mm | 84.0 | 81.0 | 84.0 | 84.0 |
| Stroke | mm | 95.9 | 77.4 | 95.9 | 95.9 |
| Compression ratio |  | 11.25 | 10.5 | 11.25 | 10.85 |
| Injection/ignition |  | Motronic ME 7.1.1 | Simos 3.3 | Motronic ME 7.1.1 | Motronic ME 7.1.1 |
| RON | min. | 98 unleaded | 95 unleaded | 98 unleaded | 95 unleaded |
| Self-diagnosis |  | yes | yes | yes | yes |
| Catalytic converter |  | yes | yes | yes | yes |
| Lambda regulation |  | yes | yes | yes | yes |
| Exhaust gas recirculation |  | no | yes | no | no |
| Turbocharged |  | no | no | no | no |

| Engine code |  | AGP | AGR | AHF |
| --- | --- | --- | --- | --- |
| Engines |  | Diesel engine | Diesel engine | Diesel engine |
| Manufactured |  | From 10.97 | From 10.97 | From 10.97 |
| Exhaust emissions fulfil |  | EU 2 | EU 2 | EU 2 |
| No. of cylinders / valves per cylinder |  | 4 / 2 | 4 / 2 | 4 / 2 |
| Capacity |  | 1.9 | 1.9 | 1.9 |
| Output | kW at rpm | 50/4200 | 66/4000 | 81/41500 |
| Torque | Nm at rpm | 133/2200 ... 2600 | 210/1900 | 235/1900 |
| Bore | Ø mm | 79.5 | 79.5 | 79.5 |
| Stroke | mm | 95.5 | 95.5 | 95.5 |
| Compression ratio |  | 19.5 | 19.5 | 19.5 |
| Injection/ignition |  | SDI | TDI | TDI |
| CN | min. | 49 | 49 | 49 |
| EPC system |  | yes | yes | yes |
| Self-diagnosis |  | yes | yes | yes |
| Catalytic converter |  | yes | yes | yes |
| Lambda regulation |  | no | no | no |
| Exhaust gas recirculation |  | yes | yes | yes |
| Turbocharged |  | no | yes | yes |
| Charge air cooling |  | no | yes | yes |

| Engine code |  | AJM | ALH | AQM |
| --- | --- | --- | --- | --- |
| Engines |  | Diesel engine | Diesel engine | Diesel engine |
| Manufactured |  | From 05.99 | from 10.97 | from 11.98 |
| Exhaust emissions fulfil |  | D3 standard | D3 standard | D3 standard |
| No. of cylinders / valves per cylinder |  | 4 / 2 | 4 / 2 | 4 / 2 |
| Capacity |  | 1.9 | 1.9 | 1.9 |
| Output | kW at rpm | 85/4000 | 66/3750 | 50/4200 |
| Torque | Nm at rpm | 285/1900 | 210/1900 | 133/2200 ... 2600 |
| Bore | Ø mm | 79.5 | 79.5 | 79.5 |
| Stroke | mm | 95.5 | 95.5 | 95.5 |
| Compression ratio |  | 18.0 | 19.5 | 19.5 |
| Injection/ignition |  | TDI (unit injector) | TDI | SDI |
| CN | min. | 49 | 49 | 49 |
| EPC system |  | yes | yes | yes |
| Self-diagnosis |  | yes | yes | yes |
| Catalytic converter |  | yes | yes | yes |
| Lambda regulation |  | no | no | no |
| Exhaust gas recirculation |  | yes | yes | yes |
| Turbocharged |  | yes | yes | no |
| Charge air cooling |  | yes | yes | no |

| Engine code |  | ARL | ASV | ASZ |
| --- | --- | --- | --- | --- |
| Engines |  | Diesel engine | Diesel engine | Diesel engine |
| Manufactured |  | from 09.00 | From 09.99 | from 11.00 |
| Exhaust emissions fulfil |  | EU 3 | D3 standard | EU 3 |
| No. of cylinders / valves per cylinder |  | 4 / 2 | 4 / 2 | 4 / 2 |
| Capacity |  | 1.9 | 1.9 | 1.9 |
| Output | kW at rpm | 110/4000 | 81/4000 | 96/4000 |
| Torque | Nm at rpm | 320/1900 | 235/1900 | 310/1900 |
| Bore | Ø mm | 79.5 | 79.5 | 79.5 |
| Stroke | mm | 95.5 | 95.5 | 95.5 |
| Compression ratio |  | 19.5 | 19.5 | 19.00 |
| Injection/ignition |  | TDI (unit injector) | TDI | TDI (unit injector) |
| CN | min. | 49 | 49 | 49 |
| EPC system |  | yes | yes | yes |
| Self-diagnosis |  | yes | yes | yes |
| Catalytic converter |  | yes | yes | yes |
| Lambda regulation |  | no | no | no |
| Exhaust gas recirculation |  | yes | yes | yes |
| Turbocharged |  | yes | yes | yes |
| Charge air cooling |  | yes | yes | yes |

| Engine code |  | ATD | AUY | AXR | BEW |
| --- | --- | --- | --- | --- | --- |
| Engines |  | Diesel engine | Diesel engine | Diesel engine | Diesel engine |
| Manufactured |  | from 02.00 | from 05.00 | from 05.01 |  |
| Exhaust emissions fulfil |  | EU 3 | EU 3 | EU 3 + D4 standard | LEV 1 |
| No. of cylinders / valves per cylinder |  | 4 / 2 | 4 / 2 | 4 / 2 | 4 / 2 |
| Capacity |  | 1.9 | 1.9 | 1.9 | 1.9 |
| Output | kW at rpm | 74/4000 | 85/4000 | 74/4000 | 74/4000 |
| Torque | Nm at rpm | 240/1800 ... 2400 | 310/1900 | 240/1800 ... 2400 | 240/1800 ... 2400 |
| Bore | Ø mm | 79.5 | 79.5 | 79.5 | 79.5 |
| Stroke | mm | 95.5 | 95.5 | 95.5 | 95.5 |
| Compression ratio |  | 19.0 | 18.0 | 19.0 | 19.0 |
| Injection/ignition |  | TDI (unit injector) | TDI (unit injector) | TDI (unit injector) | TDI (unit injector) |
| CN | min. | 49 | 49 | 49 | 49 |
| EPC system |  | yes | yes | yes | yes |
| Self-diagnosis |  | yes | yes | yes | yes |
| Catalytic converter |  | yes | yes | yes | yes |
| Lambda regulation |  | no | no | no | no |
| Exhaust gas recirculation |  | yes | yes | yes | yes |
| Turbocharged |  | yes | yes | yes | yes |
| Charge air cooling |  | yes | yes | yes | yes |