|  | Strategy # 1 Naim NAC Series Pre-amp A really simple yet effective mod for NAC series pre-amplifiers. The 62K (R2) resistor biases the 'ring of two' current source and carries transient disturbances from the +24V supply rail. Adding a 4K7 resistor plus a 47uF cap then changing R2 for a 56K to restore the equilibrium, takes the treble 'rip' out of the sound - so reducing audible aberrations by a considerable margin and making the whole performance more 'listenable'. The 'ultimate' mod added recently takes the 321 stage to entirely new heights with the addition of 'gyrator' or VBE multiplier regulation instead of the RC series dropper. The pass transistor TR6, may be any low noise NPN such as BC546 etc. Extra decoupling for the voltage stages (47R + 100uF & 120R + 47uF + 100n) and the replacement of the resistors shown by RC55 high definition components reaps dividends. Experimenters may care to substitute a J510 (3.6mA constant current diode - selected for lowest noise), instead of the 56K bias resistor. The 47uF feedback capacitor may be changed for an Oscon SG/SP series of similar value for a cleaner, crisper sound. The 10uF tantalum at the input may be changed for a 1uF PPS ( Polyphenylene Sulphide - Evox SMR ) film cap for a noticeable improvement in resolution - bass roll off is slight. The output cap may be a BC 128 series solid aluminium or an Oscon SP series, stay with10uF.................more later.......... This gentleman seems moderately pleased with his efforts: " Today I did Your recommended tweak on my Nac72. I was totally astonished to hear a completely other amp (I didn't expect that much). It's like the step when you add a hicap. In this moment I listen to a CD with old Curtis Mayfield music and it sounds nearly like from vinyl! REALLY amazing! " - Stefan Gruenenwald - Leipzig Circuit before modification NAC 32/72 - NA321 Board Layout (Thanks to Richard Hallman) Circuit after modification Hope you enjoy the improvements..!! Strategy # 2 How to make a NAP series Power Amp produce more music with less distortion. The current limiting circuitry in the NAP amplifier modules is designed to ensure that the output transistors don't produce current beyond their SOA - Safe Operating Area. It must be pointed out that this crude circuit does little or nothing to protect the output transistors against abuse such as short circuited loudspeaker leads for instance. Care is still needed even when this circuit is operational. With the NAP250 and NAP135, a current limited power supply is incorporated so this circuit is in fact redundant. With smaller NAP110, NAP140, if the manufacturers specifications are to be believed, the output devices should prove capable of shunting the entire current produced by the power supply. Unfortunately, the SOA protection limits come 'on stream' much too early and the consequence is an aberration in the reproduction of even some mild transients. Contrary to some uninformed opinion, this DOES NOT DISABLE the amp's THERMAL PROTECTION Removing the components shown, only the SOA protection is disabled. Stategy 3# 30 Volt Runniing Rail Volitage – turn it up By simply increasing the power supply voltage from the nominal 24V to around 30V DC, the performance of the NAC range of pre-amps is enhanced. Clarity, leading edge dynamics and solidity are all improved but don't ask me why..!! The components fitted by Naim are well rated & should accept the increased supply voltage without problems We have only tried this on these models: 12S, 42, 42.5, 32, 62, 72 and in each case, an improvement was noted. Any owner trying this on the above or on later models, does so at their own risk Strategy # 4 A Crossover for SBL’s Here's one for SBL Owners A more adventurous project: For those who would like to build or upgrade the SBL crossover network - here's the drawing. When building a 'clone' crossover. take the opportunity to replace the electrolytics with high grade polypropylenes. Tape wound "Air Cored" inductors will yield benefits and Meggit or Caddock resistors will add detail and 'body' to the sound. Coils and caps may be obtained from many sources; Falcon Acoustics, Wilmslow etc. If there is sufficient demand, we may be tempted to add a ready built crossover to our range Strategy # 5 Revised Signal Routing We are often asked why the factory insist on taking the signal into the power supply before diverting it off to the power amps. No other manufacturer known uses such a quirky combination of leads and routing. Over many years of experience with Naim equipment, we can think of no coherent reason why a perfectly clean signal containing minute levels of detail, should be taken by two extra connections into an area of high electromagnetic activity such as a mains power supply......... In order to question the wisdom of the factory line, let's examine their own recommendations as printed in the owner manuals: The use of a single HiCap with, as for example with the NAC82 in Figure 1 - requires that the signal is carried into the HiCap along the same lead as the supply voltage. Passing into the HiCap through this lead, the signal is then diverted to another Din socket as illustrated where it is then sent via a separate lead to the power amp(s). This, according to the explanation given by the factory, is to preserve the so called 'star earth' arrangements where the system earth is configured to appear within the power supply itself......So far so good - but read on........... Now consider Figure 2 - again lifted straight from the Naim owner manual where the use of two HiCaps is specified. As the signal passes through only one of the HiCaps, it's difficult to reconcile where exactly the notional 'star earth' will now be found as it most certainly will not appear within the second HiCap. Given this information, it appears the factory considers it perfectly legitimate to power sections of the NAC pre-amps from a totally independent source without considering 'star earthing' at all. Extrapolating even further, if this strategy is practicable, then why not go the whole hog and power up the entire shooting match from independent sources..?? Why not indeed and would you believe, it works.......... A very simple technique to improve resolving power of any Naim system is drop any idea that (a). signal and power should be combined in the same lead and (b). that the signal should pass through the power supply. Consider Figure 3 - where a re-arrangement of the leads enables the signal to be carried by a high quality dedicated screened lead whilst the DC power is entrusted to a separate conductor. IMPORTANT: use custom 3 core cables only for carrying power between power supply and pre-amp as the signal plus DC power will still be combined in the standard Naim 'snaic' and with the NAC82, the full potential of this fine pre-amp will not be realised & will result in a muddied or coloured sound. (Please see below for new product.) This strategy ensures almost complete isolation of signal and power leading to a cleaner and tighter performance devoid of any power supply induced noise. It only needs suitable DC power and signal cables and no mods to the equipment......so try it and trust your own ears to tell you if there's an improvement. Strategy # 6 Naim's rectifier system - How is it done..?? We're often asked how Naim manage to make a full wave bridge from only two diodes. To go back to the days of the valve, here's how it's done: Figure 1: Shows how to connect two separate transformers with split secondaries, for full wave rectification using just 2 diodes for either polarity. Figure 2: Shows how the same configuration may be employed when using a transformer with two totally independent secondaries and how a link may be added to form a centre zero. Don't try this with a transformer having only one secondary....the result has a high laxative factor..!! Figure 3: Shows yet another possibility - how to configure a full wave rectifying arrangement using only two double diodes. Using the same arrangement as before, each transformer secondary is used to supply an independent capacitor. As each capacitor is 'floating' with respect to any other voltage, two may be connected in series to form a very economical split rail DC supply with low impedance. Again..!! DON'T TRY THIS with a transformer having only one secondary Strategy # 7 How to set the bias (standing) current on the NAP Series: Do NOT have loudspeakers connected during this work Switch off & wait 5 mins before making or breaking any connections Always work SAFELY..!! Ensure no mains is accessible Strategy # 8 Conversion of Naims for other Mains Voltages Some dealers have been known to recommend that a new transformer is needed to convert to another voltage. In over twenty years of servicing Naim equipment, we have yet to find one item which cannot be easily converted by a simple re-arrangement of the transformer's primary windings. You'll find the extra unused wiring secreted in the harness. By following the drawing, you could save yourself a packet..!! SAFETY WARNING # 1 - Do have the work carried out by a competent technician - we need you as a customer..!! SAFETY WARNING # 2 - Don't forget to install a fuse of the correct rating..!! Firstly, for those units with the Holden and Fisher Transformer:. DIY Tips & Tricks : : Naim; Strategies. |
| --- | --- |