# Precision Audio Millivoltmeter 
source: RED Free Circuit Designs

Measures 10mV to 50Volt RMS in eight ranges

Simply connect to your Avo-meter set @ 50΅A range

Parts:

R1_____909K    1/2Watt 1% Metal Oxide Resistor

R2______90K9   1/2Watt 1% Metal Oxide Resistor

R3_______9K09  1/2Watt 1% Metal Oxide Resistor

R4_______1K01  1/2Watt 1% Metal Oxide Resistor

R5_____100K    1/4W Resistor

R6_______2M2   1/4W Resistor

R7______82K    1/4W Resistor

R8______12K    1/4W Resistor

R9_______1K2   1/4W Resistor

R10______3K3   1/4W Resistor

R11____200R    1/2W Trimmer Cermet

C1_____330nF   63V Polyester Capacitor

C2,C3__100΅F   25V Electrolytic Capacitor

C4_____220΅F   25V Electrolytic Capacitor

C5______33pF   63V Polystyrene Capacitor

C6_______2΅2   63V Electrolytic Capacitor

D1-D4___1N4148 75V 150mA Diodes

IC1_____CA3140 Op-amp

IC2_____CA3130 Op-amp

SW1_____2 poles 5 ways rotary switch

SW2_____SPDT switch

J1______RCA audio input socket

J2,J3___4mm. output sockets

B1______9V PP3 Battery

Clip for PP3 Battery

Notes:

- Connect J2 and J3 to an Avo-meter set @ 50΅A range

- Switching SW2 the four input ranges can be multiplied by 5

- Total fsd ranges are: 10mV, 50mV, 100mV, 500mV, 1V, 5V, 10V, 50V

- Set R11 to read 1V in the 1V range, with a sinewave input of 1V @ 1KHz

- Compare the reading with that of another known precision Millivoltmeter or with an oscilloscope

- The oscilloscope reading must be a sinewave of 2.828V peak to peak amplitude

- Frequency response is flat in the 20Hz-20KHz range

- If you have difficulties in finding resistor values for R1, R2, R3 & R4, you can use the following trick:
R1 = 10M + 1M in parallel
R2 = 1M + 100K in parallel
R3 = 100K + 10K in parallel
R4 = 1K2 + 6K8 in parallel
All resistors 1% tolerance