Tel:86-27-87689626 Fax:87689626

E-mail:laserlands@gmail.com

HLD808500H4T  808nm  0.5W  40℃ Laser Diodes

### ■Specifications

Wavelength:        808nm

#### Light Output:       0.5W   CW

Package Type:      TO-5(Φ9.0mm)

■Absolute Maximum Ratings (Tc=25℃)

| Parameter | Symbols | Ratings | Units |
| --- | --- | --- | --- |
| Optical Output | Po(CW) | 0.5 | W |
| Reverse Voltage | Vr | 2 | V |
| Operating Temperature | Top | -10 ~ +40 | ℃ |
| Storage Temperature | Tstg | -40 ~ +80 | ℃ |

■Electrical and optical Characteristics(Tc=25℃)

| Parameter |  | Symbols | Conditions | Min | Typ | Max | Units |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Threshold Current |  | Ith | - | - | 0.15 | 0.20 | A |
| Operating Current |  | Iop | Po=0.5W | - | 0.60 | 0.65 | A |
| Operating Voltage |  | Vop | Po=0.5W | - | 2.0 | 2.3 | Volts |
| Slope Efficiency |  | η | - | - | 0.9 | - | W/A |
|  |  |  | - |  |  |  |  |
| Beam Divergence (FWHM) | Parallel | θ‖ | Po=0.5W | 8 | 10 | 14 | deg |
|  | Perpendicular | θ⊥ | Po=0.5W | 30 | 35 | 40 | deg |
| Parallel Deviation Angle |  | △θ‖ | Po=0.5W | -3 | - | 3 | deg |
| Perpendicular Deviation Angle |  | △θ⊥ | Po=0.5W | -3 | - | 3 | deg |
| Emission Point Accuracy |  | △X,△Y,△Z | Po=0.5W | -80 | - | 80 | um |
| Lasing Wavelength |  | λp | Po=0.5W | 803 | 808 | 812 | nm |

Im is sorting by custom’s need

◎θ‖ and θ⊥ are defined as the angle within which the intensity is 50% of the peak value.