This document deals with the question, how the capacity of the toroid in a tesla coil depends on the ratio of the outer diameter of the toroid to the diameter of the tube, from which the toroid is built.

I came to this question, as same capacity is achievable with different geometries. In this document, I will show that there exists a geometric ratio of diameters, for which the capactiy has a maximum value.

As the drawing indicates, the outer toroid diameter will be d1 and the tube diameter will be d2 in the following calculations.

The formula for the capacity of the toroid I used is the following one. This can be found on many web sites of other coilers and calculates the capacity as function of d1 and d2.

C(d1,d2)=2,81,2781−d2d1π4d1−d2d225,4

First step is to introduce the ratio factor x=d2d1, which transforms the above formula as follows:

C(x)=2,81,2781−xπ4d12x−x225,4

By this way, the capacity now is only a function of d1 and the ratio of d2 and d1.

Eliminating the square root as far as possible and introducing a new variable a=2,8πd12⋅25,4 leads to the following form of the capacity formula:

C(x)=(a⋅1,2781−ax)x−x2

It is now necessary to calculate the first derivative of this term, i.e. dC(x)dx. For this reason, the first term in the product is assumed as f and the second term as g, while the term x−x2is called h.

This results in the following:

f(x)=1,2781a−ax		and	f'(x)=−a

g(x)=x−x2=h(x)	and	g'(x)=dgdh⋅dhdx=1−2x2x−x2	using the chain rule with

h(x)=x−x2			and	h'(x)=1−2x

For dC(x)dx we can write dCdx=f'⋅g+f⋅g'

This results in dC(x)dx=−ax−x2+1,2781a−ax1−2x2x−x2

For testing the extreme values, i.e. maximum capacity, this has to be set to 0. After some work, this results in the simple quadratic equation:

x2−2,77812x+1,27814=0

The solutions of this equation are:

x=d2d1=0,2909	and	x=d2d1=1,098

This second solution would result in the inner diameter to be larger than the outer diameter, which is not a toroid at all.

Result:

The toroid has maximum capacity for a given inner or outer diameter, if the ratio of inner to outer diameter is chosen to be 0,2909. For example, with a given outer diameter of 500mm the inner diameter must be 145,5mm to obtain maximum capacity, chosing other values (larger or smaller) for inner diameter results in decrease of capacity.