20 PIN MOLEX 39-29-9202 at the motherboard

20 PIN MOLEX 39-01-2200 at the cable

| Pin | Name |  | Color | Description |
| --- | --- | --- | --- | --- |
| 1 | 3.3V |  | Orange | +3.3 VDC |
| 2 | 3.3V |  | Orange | +3.3 VDC |
| 3 | COM |  | Black | Ground |
| 4 | 5V |  | Red | +5 VDC |
| 5 | COM |  | Black | Ground |
| 6 | 5V |  | Red | +5 VDC |
| 7 | COM |  | Black | Ground |
| 8 | PWR_OK |  | Gray | Power Ok (+5V & +3.3V is ok) |
| 9 | 5VSB |  | Purple | +5 VDC Standby Voltage (max 10mA) |
| 10 | 12V |  | Yellow | +12 VDC |
| 11 | 3.3V |  | Orange | +3.3 VDC |
| 12 | -12V |  | Blue | -12 VDC |
| 13 | COM |  | Black | Ground |
| 14 | /PS_ON |  | Green | Power Supply On (active low) |
| 15 | COM |  | Black | Ground |
| 16 | COM |  | Black | Ground |
| 17 | COM |  | Black | Ground |
| 18 | -5V |  | White | -5 VDC |
| 19 | 5V |  | Red | +5 VDC |
| 20 | 5V |  | Red | +5 VDC |

18 AWG is recommended for all wires except pin 11, which should be 22 AWG
For 300W configurations 16 AWG is recommended.

## Contributions

- Joakim Ögren

## Sources

- ATX Spec v2.03 at Platform Development Support