bookmark this page

| 14 pin Molex 39-01-2140 connector at the compaq | pinout for EVO D510 SFF power supply (PDP117P) or also HP-U176 WF3 part number= 243891-002 Pin; Pin Name; Description. 1; brown; +3,3volts. 2; blue; -12v. 3; violet; fan off special pin. 4; white; power on-standby. 5; green; +5volts aux. 6; white and red; fan cmd special pin ( 3.3V RS- ). 7; orange; +12volts. 8; brown; +3,3volts. 9; pink; +3,3volts aux ( 3.3v RS+ ). 10; black; ground. 11; red; +5volts. 12; black; ground. 13; red; +5volts. 14; black; ground. the lock spring is above i was starting from the up row left side. This psu is valid for almost all 14-pin Compaq evo desktops. If you are converting from an ATX Power Supply Pin 6 and 3 are not needed. The Compaq Power Supply 90W ( Compaq Square Nº 288468-001 ) has this PinLayout an fit in Compaq Deskpro EN SFF Note: Power-On on Compaq REQUIRES high signal to switch on! Connecting directly power-on with power-on on standard ATX that start on low (connect to ground) won't be enough to start Compaq motherboard. Build inverter: http://www.karosium.com/2008/05/standard-p4-atx-to-14pin-compaq-psu.html or don't connect wire 4 (white) to plug and start PSU by additional switch that connect power-on from PSU with ground. Want to add or correct something here? Edit this page! |  |
| --- | --- | --- |